Communication Procedures

Class E Airport – Non-Towered

Before Taxi:
Listen to AWOS and get advisory

While Taxiing:

Alert traffic when crossing any runway. 
 “Hemet traffic Cessna 89519 crossing runway two-three Hemet.”

Alert traffic when back-taxiing on any runway.

”Hemet traffic, Cessna 89519 back-taxiing runway two-three Hemet.” 

Note: Radio call does not relieve pilot from visually checking for traffic!

Before Takeoff:

After checking final, report when taxiing onto runway for takeoff.

“Hemet traffic, Cessna 89519 departing runway two-three to the south* Hemet.” 

*(say direction of flight or say that you will be staying in the pattern.)
Coming Back to the Airport:

Listen to AWOS or get an airport advisory from Unicom. 
 “Hemet Unicom, Cessna 89519 10 miles south inbound for landing, request airport advisories.”

Report your position to other possible traffic.

 “Hemet traffic, Cessna 89519 10 miles east inbound for landing runway two-three Hemet.” 

Entering the Pattern (Always enter on the 45):

Report entering the pattern, on the 45.  Report turning to each leg of the pattern (crosswind, downwind, base, final), and include runway of intended landing.

 “Hemet traffic, Cessna 89519 entering on the 45 for left downwind runway two-three Hemet”

 “Hemet traffic, Cessna 89519 turning left downwind runway two-three Hemet” 
After Landing and clear of the runway, report.

 “Hemet traffic, Cessna 89519 clear of runway two-three Hemet.”
Class D Airport – GA airport w/Control Tower

Arrival:

Get ATIS information (Alpha, Bravo, Charlie, etc.) 

Contact Tower ( = ATC;  = 89519) 

 “Gillespie tower, Cessna 89519, 10 miles NW, inbound for landing with Alpha.”

  “Cessna 519, report 2 mile left base runway 27L.”

 “Report 2 mile left base runway two-seven left, Cessna 519.”

 “Tower, Cessna 519 is 2 mile left base runway two-seven left.”

 “Cessna 519, you are number 2 following a Cherokee on short final. Cleared to land.”

 “Cessna 519 cleared to land, Runway two-seven left, number 2, traffic in sight (or looking for traffic).”

· “Cessna 519 turn left next taxiway, contact ground 121.7”

 “Cessna 519, left next taxiway, ground 121.7. (At this time the pilot would tune in 121.7 and listen to hear if the channel is clear)

Contact Ground ( = ATC;  = 89519)

 “Gillespie ground, Cessna 89519 off two seven left at delta four taxi to base of tower”
· “Cessna 519 cross runway 35 on taxiway Delta to base of tower”

 “Cessna 519, Roger, Cross three-five on Delta”
Departure:

Get ATIS information (Alpha, Charlie, etc.) 

Contact Ground

 “Gillespie ground, Cessna 89519 at the base of the tower, ready to taxi to Two-Seven Right with bravo.” 

   “Cessna 89519, taxi runway 27R cross 35 at Alpha four”.

 “Cessna 519, roger, taxi runway 27R cross 35 at Alpha four.”

After Run-Up, taxi to hold-short line and switch to tower frequency.


Contact Tower

 “Gillespie Tower, Cessna 89519 holding short two seven right, for a Right Downwind Departure.”


   “Cessna 519, Cleared for takeoff, right downwind departure approved.” (If told to hold short, repeat all hold short instructions.)

Class C/B Airport – Busy Airports with Control Tower

Arriving: 

Listen to ATIS 

Contact Approach Control 20 to 25 NM out. 
 “John Wayne Approach, Cessna 89519 is 25 miles south inbound for landing with Alpha.”
   “Cessna 89519, squawk 1243.”

  “Cessna 89519, radar contact 20 miles south of John Wayne Airport.”

Approach control will then monitor your progress to the airport and advise of any pertinent traffic.  Switch frequencies as requested by the controller, and say altitude each time you report in.

Contact Tower

  “Cessna 89519, contact tower on 119.9.”

 “Tower on 119.9, Cessna 519.”

 “John Wayne Tower, Cessna 89519 is with you 3 mile final runway one-nine left.”

  “Cessna 89519 you are cleared to land runway 19L.”

 “Cleared to land one-nine left, Cessna 519.”

   “Cessna 519, contact ground on 120.8.”
 “Contact ground 120.8, Cessna 519.”

Contact Ground
 “John Wayne Ground, Cessna 89519 with you clear of runway one nine left at Foxtrot , taxi to Signature Aviation”

   “Cessna 519, taxi to Signature.”

 “Taxiing to Signature, Cessna 519.”

Departing Class C/B: 

Get ATIS 

Contact Clearance Delivery. 

 “John Wayne Clearance Delivery, Cessna 89519 at Signature Aviation with Alpha, We are a Cessna 152 slant Uniform going to Gillespie Airport in San Diego.”

   “Cessna 89519, after departure turn left heading 180, climb to 2000.’ Squawk 1235, departure frequency 128.1” 

 “Turn left heading one-eight-zero after departure, climb to two thousand feet, departure frequency 128.1, squawk 1235, Cessna 519.”

  “Cessna 519, read back is correct.”

Contact Ground

 “John Wayne ground, Cessna 89519 is at Signature ready to taxi one-eight left.”

  “Cessna 89519, taxi to runway one-eight left.”

 “Taxi to runway one-eight left, Cessna 519.”

Contact Tower when ready to depart.
 “John Wayne tower, Cessna 89519 holding short runway one-eight left ready for departure.”

  “Cessna 89519, cleared for takeoff runway one-eight-left.”

 “Cleared for takeoff one-eight left, Cessna 89519.”
  “Cessna 89519, contact departure 128.1.”

 “Going to departure 128.1 Cessna 89519”
Contact Departure control when directed by tower.
 “John Wayne Departure, Cessna 89519 out of one thousand two hundred feet for two thousand.”

   “Cessna 519 radar contact, climb and maintain 3,500’, continue own navigation.”

 “Roger, climb to three-thousand five hundred and own navigation Cessna 519.”

Twenty miles out, cancel radar services if you no longer wish to have flight following
 “John Wayne Departure, Cessna 89519 is 20 miles to the South, would like to cancel radar services.”

   “Cessna 519, radar services terminated. Squawk 1200 (or VFR), Good day.”

 “Squawk VFR, Thank You, Cessna 519.”
 

Guide To All Communications:

Who you are talking to. (Twr/Gnd/Apph/Etc)

Who you are. (Model and N Number)

Where you are. (Expressed in three dimensions)

What you want.

Pg #4 – Copyright Vincent Ammann - www.cfivince.com 


